Non-Destructive SemiDisk Test for QX-10

NDQXTEST.COM  can be used to test a SemiDisk installed in  a

QX-10.   Normally,  this can be done without altering data stored

on  the SemiDisk.   The user is  cautioned,  however,  that  some

defects  could  cause  loss  of data.   It is  therefore  a  good

practice  to  backup files stored on the SemiDisk onto  a  floppy

disk before starting this test.

NDQXTEST.COM  will run equally well under CP/M,  TPM III  or

VALDOCS.   To  envoke the test,  type NDQXTEST<RETURN> after  the

system prompt.

A>NDQXTEST<RETURN>

The  program  will  then load and prompt for the number  of  512K

equivalents.   If your system uses a single 512K SemiDisk  board,

the  appropriate reponse is 1.   The reponse for a 2 Meg board is

4,  etc.   If you enter a CTRL-C, that is, type a C while holding

the  control  key down,  the program will exit to  the  operating

system.

After entering the number of 512K equivalents, the test will

begin.  The first thing the test will do is compute and store the

a 16-bit checksum word for each of the 256 (FFH) tracks.

TEST  1 tests each sector one at a time.  The current  Track

number is displayed in the upper righthand corner of the display.

The  data  contained in the sector under test is first  read  and

stored in memory.   The sector is then filled with zeros, that is

each  byte  has the Hex value 00.   Each sector location is  then

read and checked for the zero value.   Any value read other  than

zero  will cause an error which is reported to the operator as an

astrisk  on the error map in a position which cooresponse to  the

failing  memory device.   Appendix B of the SemiDisk manual  will

help determine the physical location of the failing device.

If you are testing a 2 Meg board, the error map presentation

is the same as four 512K boards,  that is, the map will be broken

into four quadrents.   Each memory device of the 2 Meg board will

actually be represented four times.

Once the sector has been checked, the old data are rewritten

and  the  test goes on to the next sector until all sectors  have

been tested with zeros.

TEST  2  is  similar  to TEST 1 except all  ones  (FFH)  are

written and checked.  Errors are reported as before.

TEST 3 tests each SemiDisk byte one at a time.  Each byte is

read, complimented, written, read again, complimented and checked

against  the  original  byte,  rewritten,  then  again  read  and

compared with the original byte.  Errors are reported as in Tests

one and two.

TEST  4 is similar to Tests 1 and 2 except data  written  in

each  sector are a binary progession.   The initial value written

in the first location of each track is 00, a 01 is written to the

second location,  then 02 and so on.   The next time through Test

4,  the value written into the first location is 01,  the next is

02, the next is 03 and so on.  As in Test 1 and 2, each sector is

checked after it is written and errors are reported.

Note  that in each test that the byte or sector under  test,

for  the  period of the test,  does not contain  data  originally

stored  there.   If the test is interrupted before original  data

can be written back into that byte or sector, data could be lost.

It is therefore very important that the test never be  terminated

with a system reset or by switching off the power.   The test may

be safely terminated any time by typing a Q for Quit.  Typing any

other  character  will cause the test to continue to the  end  of

Test 4, then enter the termination phase.

During  the termination phase,  all data are restored to the

appropriate  byte or sector.   A checksum is again generated  for

each track and compared with those checksums which were generated

and  stored  at the beginning of the test  program.   Errors  are

reported  by track number.   Unfortunately it is not possible  to

know exactly which chip caused a track-wise checksum error. If an

error  occurs,  further  testing is indicated.   After  comparing

checksums, the program will terminate and exit to the system.

VALDOCS users may experience a system crash on  exit.   This

does not invalidate the test but is inconvenient because a reboot

is  required.   This can be avoided by using the SETUP program to

redefine  the terminal as a Televideo 920 with "Bit Mode"  return

from .COM files.#############################################################################